Appendix – Computer program

%Program_System_Mn_Fe.m

%Titration of V0 mL FeSO4 (C01) + H2C2O4 (C02) + H2SO4 (C03) + CO2 (C04)

%with V mL KMnO4 (C) + CO2 (1)

clear;

clc;

global V Vmin Vstep Vmax V0 C C1 C01 C02 C03 C04 H OH pH E Kw pKw A aa bb

global H2C2O4 HC2O4 C2O4 H2CO3 HCO3 CO3 K

global logH2C2O4 logHC2O4 logC2O4 logH2CO3 logHCO3 logCO3 logK

global Mn7O4 Mn6O4 HSO4 SO4

global logMn7O4 logMn6O4 logHSO4 logSO4

global Mn3 Mn3OH Mn3C2O4 Mn3C2O42 Mn3C2O43

global logMn3 logMn3OH logMn3C2O4 logMn3C2O42 logMn3C2O43

global Mn2 Mn2OH Mn2SO4 Mn2C2O4 Mn2C2O42

global logMn2 logMn2OH logMn2SO4 logMn2C2O4 logMn2C2O42

global Fe2 Fe2OH Fe2SO4 Fe2C2O42 Fe2C2O43

global logFe2 logFe2OH logFe2SO4 logFe2C2O42 logFe2C2O43

global Fe3 Fe3OH Fe3OH2 Fe32OH2 Fe3SO4 Fe3SO42 Fe3C2O4 Fe3C2O42 Fe3C2O43

global logFe3 logFe3OH logFe3OH2 logFe32OH2 logFe3SO4 logFe3SO42 logFe3C2O4

global logFe3C2O42 logFe3C2O43

global pr1 pr2 logpr1 logpr2

%*****ENTERING THE NAME OF OUTPUT FILE*****

disp('You can specify the file name and path')

NP=input('Enter the file name ','s');

%****CONCENTRATIONS OF STARTING SOLUTIONS AND VOLUMES ADDED*****

C01=input('Enter the concentration of FeSO4 C01=');

C02=input('Enter the concentration of H2C2O4 C02=');

C03=input('Enter the concentration of H2SO4 C03=');

C04=input('Enter the concentration of CO2 C04=');

V0=input('Enter the volume of titranD V0=');

C=input('Enter the concentration of titranT KMnO4 C=');

C1=input('Enter the concentration of CO2 C1=');

Vmin=input('Enter the minimal volume of titranT Vmin=');

Vstep=input('Enter the portion of titranT Vstep=');

Vmax=input('Enter the maximal volume of titranT Vmax=');

%*************THE END OF SEGMENT**********************

%****STARTING VALUES FOR FUNDAMENTAL VARIABLES*****

disp('Starting Values');

x0(1)=input('Enter starting value of pH=');

x0(2)=input('Enter starting value of E=');

x0(3)=input('Enter starting value of pMn2=');

x0(4)=input('Enter starting value of pFe2=');

x0(5)=input('Enter starting value of pH2C2O4=');

x0(6)=input('Enter starting value of pSO4=');

%*************THE END OF SEGMENT**********************

%************THE OPEN OUTPUT FILE*********************

[fid]=fopen(NP,'w');

fprintf(fid,'\t\tResults of simulation\n\n');

fprintf(fid,'\t   C01= %11.6f   \n',C01);

fprintf(fid,'\t   C02= %11.6f   \n',C02);

fprintf(fid,'\t   C03= %11.6f   \n',C03);

fprintf(fid,'\t   C04= %11.6f   \n',C04);

fprintf(fid,'\t    V0= %11.6f   \n',V0);

fprintf(fid,'\t     C= %11.6f   \n',C);

fprintf(fid,'\t    C1= %11.6f   \n',C1);

fprintf(fid,'\t  Vmin= %11.6f   \n',Vmin);

fprintf(fid,'\t Vstep= %11.6f   \n',Vstep);

fprintf(fid,'\t  Vmax= %11.6f   \n\n',Vmax);

fprintf(fid,'     V                  ');

fprintf(fid,'pH                ');

fprintf(fid,'SS                    ');

fprintf(fid,'E                 ');

fprintf(fid,'logMn2             ');

fprintf(fid,'logMn2OH            ');

fprintf(fid,'logMn2SO4             ');

fprintf(fid,'logpr2               ');

fprintf(fid,'logMn2C2O4             ');

fprintf(fid,'logMn2C2O42             ');

fprintf(fid,'logMn3                 ');

fprintf(fid,'logMn3OH              ');

fprintf(fid,'logMn3C2O4             ');

fprintf(fid,'logMn3C2O42             ');

fprintf(fid,'logMn3C2O43            ');

fprintf(fid,'logMn6O4              ');

fprintf(fid,'logMn7O4             ');

fprintf(fid,'logFe2              ');

fprintf(fid,'logFe2OH             ');

fprintf(fid,'logFe2SO4                 ');

fprintf(fid,'logpr1                ');

fprintf(fid,'logFe2C2O42          ');

fprintf(fid,'logFe2C2O43             ');

fprintf(fid,'logFe3             ');

fprintf(fid,'logFe3OH            ');

fprintf(fid,'logFe3OH2             ');

fprintf(fid,'logFe32OH2            ');

fprintf(fid,'logFe3SO4             ');

fprintf(fid,'logFe3SO42             ');

fprintf(fid,'logFe3C2O4             ');

fprintf(fid,'logFe3C2O42           ');

fprintf(fid,'logFe3C2O43           ');

fprintf(fid,'logH2CO3             ');

fprintf(fid,'logHCO3            ');

fprintf(fid,'logCO3             ');

fprintf(fid,'logH2C2O4            ');

fprintf(fid,'logHC2O4             ');

fprintf(fid,'logC2O4             ');

fprintf(fid,'logHSO4             ');

fprintf(fid,'logSO4             ');

fprintf(fid,'logK\n');

%*******************THE LOOP OF TiTRATION COMPUTING********************

for V=Vmin:Vstep:Vmax

aa=0;

bb=0;

options=optimset('Display','iter','MaxIter',5000,'MaxFunEvals',...

5000,'TolFun',1.0e-50,'TolX',1.e-50);

[x,fval,output]=fsolve(@Function_System_Mn_Fe,x0,options);

if Fe2*C2O4>=10^-6.7 && Mn2*C2O4>=10^-5.3

aa=1;

bb=1;

options=optimset('Display','iter','MaxIter',5000,'MaxFunEvals',...

5000,'TolFun',1.0e-50,'TolX',1.e-50);

[x,fval,output]=fsolve(@Function_System_Mn_Fe,x0,options);

elseif Fe2*C2O4>=10^-6.7

aa=1;

bb=0;

options=optimset('Display','iter','MaxIter',5000,'MaxFunEvals',...

5000,'TolFun',1.0e-50,'TolX',1.e-50);

[x,fval,output]=fsolve(@Function_System_Mn_Fe,x0,options);

elseif Mn2*C2O4>=10^-5.3

aa=0;

bb=1;

options=optimset('Display','iter','MaxIter',5000,'MaxFunEvals', ...

5000,'TolFun',1.0e-50,'TolX',1.e-50);

[x,fval,output]=fsolve(@Function_System_Mn_Fe,x0,options);

end;

SS=sum(sum(fval.*fval));

%*************OUTPUT DATA OF TITRATION*************************

fprintf(fid,'%11.6f        ',V);

fprintf(fid,'%11.6f        ',pH);

fprintf(fid,'%6.3e         ',SS);

fprintf(fid,'%11.6f        ',E);

fprintf(fid,'%11.6f        ',logMn2);

fprintf(fid,'%11.6f        ',logMn2OH);

fprintf(fid,'%11.6f        ',logMn2SO4);

fprintf(fid,'%11.6f        ',logpr2);

fprintf(fid,'%11.6f        ',logMn2C2O4);

fprintf(fid,'%11.6f        ',logMn2C2O42);

fprintf(fid,'%11.6f        ',logMn3);

fprintf(fid,'%11.6f        ',logMn3OH);

fprintf(fid,'%11.6f        ',logMn3C2O4);

fprintf(fid,'%11.6f        ',logMn3C2O42);

fprintf(fid,'%11.6f        ',logMn3C2O43);

fprintf(fid,'%11.6f        ',logMn6O4);

fprintf(fid,'%11.6f        ',logMn7O4);

fprintf(fid,'%11.6f        ',logFe2);

fprintf(fid,'%11.6f        ',logFe2OH);

fprintf(fid,'%11.6f        ',logFe2SO4);

fprintf(fid,'%11.6f        ',logpr1);

fprintf(fid,'%11.6f        ',logFe2C2O42);

fprintf(fid,'%11.6f        ',logFe2C2O43);

fprintf(fid,'%11.6f        ',logFe3);

fprintf(fid,'%11.6f        ',logFe3OH);

fprintf(fid,'%11.6f        ',logFe3OH2);

fprintf(fid,'%11.6f        ',logFe32OH2);

fprintf(fid,'%11.6f        ',logFe3SO4);

fprintf(fid,'%11.6f        ',logFe3SO42);

fprintf(fid,'%11.6f        ',logFe3C2O4);

fprintf(fid,'%11.6f        ',logFe3C2O42);

fprintf(fid,'%11.6f        ',logFe3C2O43);

fprintf(fid,'%11.6f        ',logH2CO3);

fprintf(fid,'%11.6f        ',logHCO3);

fprintf(fid,'%11.6f        ',logCO3);

fprintf(fid,'%11.6f        ',logH2C2O4);

fprintf(fid,'%11.6f        ',logHC2O4);

fprintf(fid,'%11.6f        ',logC2O4);

fprintf(fid,'%11.6f        ',logHSO4);

fprintf(fid,'%11.6f        ',logSO4);

fprintf(fid,'%11.6f\n'   ,logK);

x0=x;

x0(3)=log10(C.*V./(V0+V));

%**************THE END OF OUTPUT DATA*************************

end;

%*******************THE END OF LOOP*****************************

fclose(fid);

%*******************CLOSE THE OUTPUT FILE*****************************

type(NP)

disp('The end of program');

function F = Function_System_Mn_Fe(x)

global V Vmin Vstep Vmax V0 C C1 C01 C02 C03 C04 H OH pH E Kw pKw A aa bb

global H2C2O4 HC2O4 C2O4 H2CO3 HCO3 CO3 K

global logH2C2O4 logHC2O4 logC2O4 logH2CO3 logHCO3 logCO3 logK

global Mn7O4 Mn6O4 HSO4 SO4

global logMn7O4 logMn6O4 logHSO4 logSO4

global Mn3 Mn3OH Mn3C2O4 Mn3C2O42 Mn3C2O43

global logMn3 logMn3OH logMn3C2O4 logMn3C2O42 logMn3C2O43

global Mn2 Mn2OH Mn2SO4 Mn2C2O4 Mn2C2O42

global logMn2 logMn2OH logMn2SO4 logMn2C2O4 logMn2C2O42

global Fe2 Fe2OH Fe2SO4 Fe2C2O42 Fe2C2O43

global logFe2 logFe2OH logFe2SO4 logFe2C2O42 logFe2C2O43

global Fe3 Fe3OH Fe3OH2 Fe32OH2 Fe3SO4 Fe3SO42 Fe3C2O4 Fe3C2O42 Fe3C2O43

global logFe3 logFe3OH logFe3OH2 logFe32OH2 logFe3SO4 logFe3SO42 logFe3C2O4

global logFe3C2O42 logFe3C2O43

global pr1 pr2 logpr1 logpr2

pH=x(1);

E=x(2);

if bb==0

Mn2=10.^-x(3);

pr2=0;

else

pr2=10.^-x(3);

end;

if aa==0

Fe2=10.^-x(4);

pr1=0;

else

pr1=10.^-x(4);

end;

H2C2O4=10.^-x(5);

SO4=10.^-x(6);

H=10.^-pH;

pKw=14;

Kw=10.^-14;

OH=Kw./H;

A=16.92;

ZFe=26;

ZMn=25;

ZC=6;

H2CO3=H2C2O4.^0.5.*10.^(A.*(E+0.396)+pH);

CO3=H2CO3.*10.^(2.*pH-16.4);

HCO3=10.^10.1.*H.*CO3;

C2O4=H2C2O4.*10.^(2.*pH-5.2);

HC2O4=10.^3.8.*H.*C2O4;

if bb==1

Mn2=10.^-5.3./C2O4;

end;

Mn7O4=Mn2.*10.^(5.*A.*(E-1.507)+8.*pH);

Mn6O4=Mn2.*10.^(4.*A.*(E-1.743)+8.*pH);

Mn3=Mn2.*10.^(A.*(E-1.509));

if aa==1

Fe2=10.^-6.7./C2O4;

end;

Fe3=Fe2.*10.^(A.*(E-0.771));

HSO4=10.^1.8*H*SO4;

Fe2OH=10.^4.5.*Fe2.*OH;

Fe2SO4=10.^2.3.*Fe2.*SO4;

Fe2C2O42=10.^4.52.*Fe2.*C2O4.^2;

Fe2C2O43=10.^5.22.*Fe2.*C2O4.^3;

Fe3OH=10.^11.0.*Fe3.*OH;

Fe3OH2=10.^21.7.*Fe3.*OH.^2;

Fe32OH2=10.^25.1.*Fe3.^2.*OH.^2;

Fe3SO4=10.^4.18.*Fe3.*SO4;

Fe3SO42=10.^7.4.*Fe3.*SO4.^2;

Fe3C2O4=10.^7.53.*Fe3.*C2O4;

Fe3C2O42=10.^13.64.*Fe3.*C2O4.^2;

Fe3C2O43=10.^18.46.*Fe3.*C2O4.^3;

Mn2OH=10.^3.4.*Mn2.*OH;

Mn2SO4=10.^2.28.*Mn2.*SO4;

Mn2C2O4=10.^3.82.*Mn2.*C2O4;

Mn2C2O42=10.^5.25.*Mn2.*C2O4.^2;

Mn3OH=10.^14.2.*Mn3.*OH;

Mn3C2O4=10.^9.98.*Mn3.*C2O4;

Mn3C2O42=10.^16.57.*Mn3.*C2O4.^2;

Mn3C2O43=10.^19.42.*Mn3.*C2O4.^3;

K=C.*V./(V0+V);

%Charge balance

F=[(H-OH+K-HSO4-2.*SO4-HC2O4-2.*C2O4-HCO3-2.*CO3-Mn7O4-2.*Mn6O4...

+3*Mn3+2.*Mn3OH+Mn3C2O4-Mn3C2O42-3.*Mn3C2O43+2.*Mn2+Mn2OH...

-2*Mn2C2O42+2.*Fe2+Fe2OH-2.*Fe2C2O42-4.*Fe2C2O43...

+3.*Fe3+2.*Fe3OH+Fe3OH2+4.*Fe32OH2+Fe3SO4-Fe3SO42...

+Fe3C2O4-Fe3C2O42-3.*Fe3C2O43);

%Concentration balance of Mn

(Mn7O4+Mn6O4+Mn3+Mn3OH+Mn3C2O4+Mn3C2O42+Mn3C2O43...

+Mn2+Mn2OH+Mn2SO4+Mn2C2O4+Mn2C2O42+bb.*pr2-C.*V/(V0+V));

%Concentration balance of Fe

(Fe2+Fe2OH+Fe2SO4+Fe2C2O42+Fe2C2O43+Fe3+Fe3OH...

+Fe3OH2+2.*Fe32OH2+Fe3SO4+Fe3SO42+Fe3C2O4+Fe3C2O42+Fe3C2O43+aa.*pr1...

-C01.*V0./(V0+V));

%Concentration balance of C

(2.*H2C2O4+2.*HC2O4+2.*C2O4+H2CO3+HCO3+CO3+2.*Mn2C2O4+4.*Mn2C2O42...

+2.*Mn3C2O4+4.*Mn3C2O42+6.*Mn3C2O43+4.*Fe2C2O42+6.*Fe2C2O43...

+2.*Fe3C2O4+4.*Fe3C2O42+6.*Fe3C2O43+2.*aa.*pr1+2.*bb.*pr2...

-(2.*C02.*V0+C04.*V0+C1.*V)./(V0+V));

%Concentration balance of S

(HSO4+SO4+Mn2SO4+Fe2SO4+Fe3SO4+2.*Fe3SO42-(C01.*V0+C03.*V0)./(V0+V));

%Electron balance

((ZMn-7).*Mn7O4+(ZMn-6).*Mn6O4+(ZMn-3).*(Mn3+Mn3OH)+…

(ZMn-2).*(Mn2+Mn2OH+Mn2SO4)...

+(ZFe-2).*(Fe2+Fe2OH+Fe2SO4)...

+(ZFe-3).*(Fe3+Fe3OH+Fe3OH2+Fe32OH2+Fe3SO4+Fe3SO42)...

+(ZC-4).*(H2CO3+HCO3+CO3)+(2.*ZC-6).*(H2C2O4+HC2O4+C2O4)...

+(ZMn-3+2.*(ZC-3)).*Mn3C2O4+(ZMn-3+4.*(ZC-3)).*Mn3C2O42...

+(ZMn-3+6.*(ZC-3)).*Mn3C2O43+(ZMn-2+2.*(ZC-3)).*Mn2C2O4...

+(ZMn-2+4.*(ZC-3)).*Mn2C2O42+(ZFe-3+2.*(ZC-3)).*Fe3C2O4...

+(ZFe-3+4.*(ZC-3)).*Fe3C2O42+(ZFe-3+6.*(ZC-3)).*Fe3C2O43...

+(ZFe-2+4.*(ZC-3)).*Fe2C2O42+(ZFe-2+6.*(ZC-3)).*Fe2C2O43...

+(ZFe-2+2.*ZC-6).*aa.*pr1+(ZMn-2+2.*ZC-6).*bb.*pr2 ...

-((ZFe-2).*C01.*V0+(2.*ZC-6).*C02.*V0...

+(ZC-4).*(C04.*V0+C1.*V)+(ZMn-7).*C.*V)/(V0+V))];

logMn2=log10(Mn2);

logMn2OH=log10(Mn2OH);

logMn2SO4=log10(Mn2SO4);

logpr2=log10(pr2);

logMn2C2O4=log10(Mn2C2O4);

logMn2C2O42=log10(Mn2C2O42);

logMn3=log10(Mn3);

logMn3OH=log10(Mn3OH);

logMn3C2O4=log10(Mn3C2O4);

logMn3C2O42=log10(Mn3C2O42);

logMn3C2O43=log10(Mn3C2O43);

logMn6O4=log10(Mn6O4);

logMn7O4=log10(Mn7O4);

logFe2=log10(Fe2);

logFe2OH=log10(Fe2OH);

logFe2SO4=log10(Fe2SO4);

logpr1=log10(pr1);

logFe2C2O42=log10(Fe2C2O42);

logFe2C2O43=log10(Fe2C2O43);

logFe3=log10(Fe3);

logFe3OH=log10(Fe3OH);

logFe3OH2=log10(Fe3OH2);

logFe32OH2=log10(Fe32OH2);

logFe3SO4=log10(Fe3SO4);

logFe3SO42=log10(Fe3SO42);

logFe3C2O4=log10(Fe3C2O4);

logFe3C2O42=log10(Fe3C2O42);

logFe3C2O43=log10(Fe3C2O43);

logH2CO3=log10(H2CO3);

logHCO3=log10(HCO3);

logCO3=log10(CO3);

logH2C2O4=log10(H2C2O4);

logHC2O4=log10(HC2O4);

logC2O4=log10(C2O4);

logHSO4=log10(HSO4);

logSO4=log10(SO4);

logK=log10(K);

function F = Function_MnO4_H2C2O4(x)

global V Vmin Vstep Vmax V0 C C0 Ca fi H OH pH E Kw pKw A aa

global H2C2O4 HC2O4 C2O4 H2CO3 HCO3 CO3 K

global logH2C2O4 logHC2O4 logC2O4 logH2CO3 logHCO3 logCO3 logK

global Mn7O4 Mn6O4 HSO4 SO4

global logMn7O4 logMn6O4 logHSO4 logSO4

global Mn3 Mn3OH Mn3C2O4 Mn3C2O42 Mn3C2O43

global logMn3 logMn3OH logMn3C2O4 logMn3C2O42 logMn3C2O43

global Mn2 Mn2OH Mn2SO4 Mn2C2O4 Mn2C2O42

global logMn2 logMn2OH logMn2SO4 logMn2C2O4 logMn2C2O42

global pr logpr q logq

pH=x(1);

E=x(2);

if aa==0

Mn2=10.^-x(3);

pr=0;

else

pr=10.^-x(3);

end;

H2C2O4=10.^-x(4);

SO4=10.^-x(5);

H=10.^-pH;

pKw=14;

Kw=10.^-14;

OH=Kw./H;

A=16.92;

ZMn=25;

ZC=6;

Kso=10.^-5.3;

H2CO3=H2C2O4.^0.5.*10.^(A.*(E+0.396)+pH);

CO3=H2CO3.*10.^(2.*pH-16.4);

HCO3=10.^10.1.*H.*CO3;

C2O4=H2C2O4.*10.^(2.*pH-5.2);

HC2O4=10.^3.8.*H.*C2O4;

if aa==1

Mn2=Kso./C2O4;

end;

Mn7O4=Mn2.*10.^(5.*A.*(E-1.507)+8.*pH);

Mn6O4=Mn2.*10.^(4.*A.*(E-1.743)+8.*pH);

Mn3=Mn2.*10.^(A.*(E-1.509));

HSO4=10.^1.8*H*SO4;

Mn2OH=10.^3.4.*Mn2.*OH;

Mn2SO4=10.^2.28.*Mn2.*SO4;

Mn2C2O4=10.^3.82.*Mn2.*C2O4;

Mn2C2O42=10.^5.25.*Mn2.*C2O4.^2;

Mn3OH=10.^14.2.*Mn3.*OH;

Mn3C2O4=10.^9.98.*Mn3.*C2O4;

Mn3C2O42=10.^16.57.*Mn3.*C2O4.^2;

Mn3C2O43=10.^19.42.*Mn3.*C2O4.^3;

K=C.*V./(V0+V);

%Charge balance

F=[(H-OH+K-HSO4-2.*SO4-HC2O4-2.*C2O4-HCO3-2.*CO3-Mn7O4-2.*Mn6O4...

+3.*Mn3+2.*Mn3OH+Mn3C2O4-Mn3C2O42-3.*Mn3C2O43+2.*Mn2+Mn2OH...

-2.*Mn2C2O42);

%Concentration balance of Mn

(Mn7O4+Mn6O4+Mn3+Mn3OH+Mn3C2O4+Mn3C2O42+Mn3C2O43...

+Mn2+Mn2OH+Mn2SO4+Mn2C2O4+Mn2C2O42+aa.*pr-C.*V/(V0+V));

%Concentration balance of C

(2.*H2C2O4+2.*HC2O4+2.*C2O4+H2CO3+HCO3+CO3+2.*Mn2C2O4...

+4.*Mn2C2O42+2.*Mn3C2O4+4.*Mn3C2O42+6.*Mn3C2O43...

+2.*aa.*pr-2.*C0.*V0./(V0+V));

%Concentration balance of S

(HSO4+SO4+Mn2SO4-(C0.*V0+Ca.*V0)./(V0+V));

%Electron balance

((ZMn-7).*Mn7O4+(ZMn-6).*Mn6O4+(ZMn-3).*(Mn3+Mn3OH)...

+(ZMn-2).*(Mn2+Mn2OH+Mn2SO4)+(ZC-4).*(H2CO3+HCO3+CO3)...

+(2.*ZC-6).*(H2C2O4+HC2O4+C2O4)+(ZMn-3+2.*ZC-6).*Mn3C2O4...

+(ZMn-3+4.*ZC-12).*Mn3C2O42+(ZMn-3+6.*ZC-18).*Mn3C2O43...

+(ZMn-2+2.*ZC-6).*Mn2C2O4+(ZMn-2+4.*ZC-12).*Mn2C2O42+...

+(ZMn-2+2.*ZC-6).*aa.*pr ...

-((2.*ZC-6).*C0.*V0+(ZMn-7).*C.*V)./(V0+V))];

q=Mn2.*C2O4./Kso;

logMn2=log10(Mn2);

logMn2OH=log10(Mn2OH);

logMn2SO4=log10(Mn2SO4);

logq=log10(q);

logpr=log10(pr);

logMn2C2O4=log10(Mn2C2O4);

logMn2C2O42=log10(Mn2C2O42);

logMn3=log10(Mn3);

logMn3OH=log10(Mn3OH);

logMn3C2O4=log10(Mn3C2O4);

logMn3C2O42=log10(Mn3C2O42);

logMn3C2O43=log10(Mn3C2O43);

logMn6O4=log10(Mn6O4);

logMn7O4=log10(Mn7O4);

logH2CO3=log10(H2CO3);

logHCO3=log10(HCO3);

logCO3=log10(CO3);

logH2C2O4=log10(H2C2O4);

logHC2O4=log10(HC2O4);

logC2O4=log10(C2O4);

logHSO4=log10(HSO4);

logSO4=log10(SO4);

logK=log10(K);